CSA1203 ASSESSMENT

NAME: DHEVAGOPALAN B

REG NO: 192512134

Design and Analysis of an Energy-Efficient Pipelined Processor with Hierarchical Memory System for Smart Industrial Monitoring

System Architecture / Design

Calculations of

- CPI

- Clock cycles

- Execution time

- Speedup

- Cache hit/miss rate

- AMAT

1. CPI (Cycles Per Instruction)

Formula

CPI=Total Clock CyclesTotal Instructions Executed

Assume the optimized processor executes 1,000,000 instructions in 1,200,000 clock cycles.

CPI=1,200,0001,000,000
CPI=1.2

Comparison

| Processor Design | CPI |
| --- | --- |
| Non-Pipelined Processor | 4.5 |
| Basic Pipelined Processor | 1.8 |
| Optimized Pipelined Processor | 1.2 |

Result

The optimized pipelined processor has the lowest CPI of 1.2, indicating better instruction throughput.

2. Clock Cycle Time

Formula

Clock Cycle Time=1Clock Frequency

Given:

Clock Frequency=1GHz
Clock Cycle Time=11×109
Clock Cycle Time=1ns

Result

Each processor clock cycle takes 1 nanosecond.

3. Total Clock Cycles

Formula

Total Clock Cycles=Instruction Count×CPI

Non-Pipelined Processor

=1,000,000×4.5
4,500,000 cycles

Basic Pipeline

=1,000,000×1.8
1,800,000 cycles

Optimized Pipeline

=1,000,000×1.2
1,200,000 cycles

Comparison

| Architecture | Instruction Count | CPI | Total Clock Cycles |
| --- | --- | --- | --- |
| Non-Pipelined | 1,000,000 | 4.5 | 4,500,000 |
| Basic Pipeline | 1,000,000 | 1.8 | 1,800,000 |
| Optimized Pipeline | 1,000,000 | 1.2 | 1,200,000 |

4. CPU Execution Time

Formula

CPU Execution Time=Total Clock Cycles×Clock Cycle Time

Alternatively:

CPU Execution Time=Instruction Count×CPI×Clock Cycle Time

A. Non-Pipelined Processor

=1,000,000×4.5×1ns
=4,500,000ns
4.5ms

B. Basic Pipelined Processor

=1,000,000×1.8×1ns
1.8ms

C. Optimized Pipelined Processor

=1,000,000×1.2×1ns
1.2ms

Final Comparison

| Processor | Execution Time |
| --- | --- |
| Non-Pipelined | 4.5 ms |
| Basic Pipeline | 1.8 ms |
| Optimized Pipeline | 1.2 ms |

Result

The optimized pipelined processor completes the same workload in the shortest time.

5. Speedup Calculation

Formula

Speedup=Execution Time of Old SystemExecution Time of New System

Comparing non-pipelined and optimized pipelined processors:

Speedup=4.51.2
Speedup=3.75

Result

The optimized pipeline is 3.75× faster

than the non-pipelined processor.

6. Cache Hit Rate and Miss Rate

Given:

- Total Memory Accesses = 10,000

- Cache Hits = 9,500

- Cache Misses = 500

Cache Hit Rate

Formula

Hit Rate=Cache HitsTotal Memory Accesses×100
=950010000×100
Hit Rate=95%

Cache Miss Rate

Formula

Miss Rate=Cache MissesTotal Memory Accesses×100
=50010000×100
Miss Rate=5%

Result

| Parameter | Value |
| --- | --- |
| Total Accesses | 10,000 |
| Cache Hits | 9,500 |
| Cache Misses | 500 |
| Hit Rate | 95% |
| Miss Rate | 5% |

A high cache hit rate reduces expensive accesses to lower memory levels.

7. AMAT (Average Memory Access Time)

For a hierarchical memory system:

Formula

AMAT=L1 Hit Time+L1 Miss Rate×Miss Penalty

First calculate the L1 miss penalty.

L1 Miss Penalty

L1 Miss Penalty=L2 Access Time+L2 Miss Rate×Main Memory Access Time

Substitute:

=5+0.10×100
=5+10
L1 Miss Penalty=15ns

Calculate AMAT

Given:

- L1 Hit Time = 1 ns

- L1 Miss Rate = 5% = 0.05

- L1 Miss Penalty = 15 ns

AMAT=1+0.05×15
=1+0.75

GRAPH COMPARISON

ROUGH GRAPH

CPI Comparison

CPI

4.5 ████████████████████ Non-Pipeline

1.8 ████████ Basic Pipeline

1.2 █████ Forwarding Pipeline

Graph 2: Execution Time Comparison

4.5ms ████████████████████ Non-Pipeline

1.8ms ████████ Basic Pipeline

1.2ms █████ Optimized Pipeline