CSA1203 ASSESSMENT

NAME: DHEVAGOPALAN B

REG NO: 192512134

Design and Analysis of an Energy-Efficient Pipelined Processor with Hierarchical Memory System for Smart Industrial Monitoring

System Architecture / Design

Calculations of

- CPI

- Clock cycles

- Execution time

- Speedup

- Cache hit/miss rate

- AMAT

1. CPI (Cycles Per Instruction)

Formula

CPI=Total Clock CyclesTotal Instructions Executed

Assume the optimized processor executes 1,000,000 instructions in 1,200,000 clock cycles.

CPI=1,200,0001,000,000
CPI=1.2

Comparison

| Processor Design | CPI |
| --- | --- |
| Non-Pipelined Processor | 4.5 |
| Basic Pipelined Processor | 1.8 |
| Optimized Pipelined Processor | 1.2 |

Result

The optimized pipelined processor has the lowest CPI of 1.2, indicating better instruction throughput.

2. Clock Cycle Time

Formula

Clock Cycle Time=1Clock Frequency

Given:

Clock Frequency=1GHz
Clock Cycle Time=11×109
Clock Cycle Time=1ns

Result

Each processor clock cycle takes 1 nanosecond.

3. Total Clock Cycles

Formula

Total Clock Cycles=Instruction Count×CPI

Non-Pipelined Processor

=1,000,000×4.5
4,500,000 cycles

Basic Pipeline

=1,000,000×1.8
1,800,000 cycles

Optimized Pipeline

=1,000,000×1.2
1,200,000 cycles

Comparison

| Architecture | Instruction Count | CPI | Total Clock Cycles |
| --- | --- | --- | --- |
| Non-Pipelined | 1,000,000 | 4.5 | 4,500,000 |
| Basic Pipeline | 1,000,000 | 1.8 | 1,800,000 |
| Optimized Pipeline | 1,000,000 | 1.2 | 1,200,000 |

4. CPU Execution Time

Formula

CPU Execution Time=Total Clock Cycles×Clock Cycle Time

Alternatively:

CPU Execution Time=Instruction Count×CPI×Clock Cycle Time

A. Non-Pipelined Processor

=1,000,000×4.5×1ns
=4,500,000ns
4.5ms

B. Basic Pipelined Processor

=1,000,000×1.8×1ns
1.8ms

C. Optimized Pipelined Processor

=1,000,000×1.2×1ns
1.2ms

Final Comparison

| Processor | Execution Time |
| --- | --- |
| Non-Pipelined | 4.5 ms |
| Basic Pipeline | 1.8 ms |
| Optimized Pipeline | 1.2 ms |

Result

The optimized pipelined processor completes the same workload in the shortest time.

5. Speedup Calculation

Formula

Speedup=Execution Time of Old SystemExecution Time of New System

Comparing non-pipelined and optimized pipelined processors:

Speedup=4.51.2
Speedup=3.75

Result

The optimized pipeline is 3.75× faster

than the non-pipelined processor.

6. Cache Hit Rate and Miss Rate

Given:

- Total Memory Accesses = 10,000

- Cache Hits = 9,500

- Cache Misses = 500

Cache Hit Rate

Formula

Hit Rate=Cache HitsTotal Memory Accesses×100
=950010000×100
Hit Rate=95%

Cache Miss Rate

Formula

Miss Rate=Cache MissesTotal Memory Accesses×100
=50010000×100
Miss Rate=5%

Result

| Parameter | Value |
| --- | --- |
| Total Accesses | 10,000 |
| Cache Hits | 9,500 |
| Cache Misses | 500 |
| Hit Rate | 95% |
| Miss Rate | 5% |

A high cache hit rate reduces expensive accesses to lower memory levels.

7. AMAT (Average Memory Access Time)

For a hierarchical memory system:

Formula

AMAT=L1 Hit Time+L1 Miss Rate×Miss Penalty

First calculate the L1 miss penalty.

L1 Miss Penalty

L1 Miss Penalty=L2 Access Time+L2 Miss Rate×Main Memory Access Time

Substitute:

=5+0.10×100
=5+10
L1 Miss Penalty=15ns

Calculate AMAT

Given:

- L1 Hit Time = 1 ns

- L1 Miss Rate = 5% = 0.05

- L1 Miss Penalty = 15 ns

AMAT=1+0.05×15
=1+0.75

GRAPH COMPARISON

ROUGH GRAPH

CPI Comparison

CPI

4.5 ████████████████████ Non-Pipeline

1.8 ████████ Basic Pipeline

1.2 █████ Forwarding Pipeline

Graph 2: Execution Time Comparison

4.5ms ████████████████████ Non-Pipeline

1.8ms ████████ Basic Pipeline

1.2ms █████ Optimized Pipeline

Performance Comparison & Engineering Recommendation

Different processor organizations are compared based on performance, power consumption, and hardware complexity.

Pipelining with forwarding and hazard detection provides improved throughput with moderate complexity.

Complex out-of-order and speculative techniques are avoided where their additional power is not justified.

The proposed cache hierarchy reduces memory access latency and improves overall system performance.

Therefore, the optimized pipelined architecture is recommended for energy-efficient industrial monitoring.

SIMULATION